# AVERERA PROBLEM STATEMENT REPORT

July 27, 2025

# PROBLEM STATEMENT 2

## Circuit Design and Component Connections

|  | The components used here are taken similar way from the first problem statement’s DC motor controller, but this for 2 DC motors, thus making us use 2 DC motors with encoder, 4 P-MOS (IRF9Z34N), 4 N-MOS (IRLZ44N), 4 NPN transistors (2N2219), three-way switch (SW-ROT-3), potentiometer to control steering angle this time, serial monitor and resistors with resistances 220 ohms and 10k ohms. The connections for the making of H-Bridge would be the same as in the first problem statement using MOSFETs and its already explained in first problem statement’s report. Here, the difference Is the number of pins used but working is the same of the controller as before. The choice of each model and components had been explained in previous report. Where this time I have removed the work of encoders as it wasn’t mentioned in this question to show the position (angle) of motors and instead focused on the speeds, as it would need to read two encoder pin reading interrupts and thus, I felt to not show them here due to error, instead RPM of motor would be shown down below the motor by default. |
| --- | --- |

## Explanation of Concepts Used

|  | After referring articles in internet like IEEE Xplore, ResearchGate and others I found that the below geometry known as Ackermann-Jeantad geometry. In this electronic differential, it allows the left and right wheels move with different speeds with respect to the turn and the steering angle (delta) dictating an optimum turning radius. This condition is necessary when the vehicle turns in curve, as the external wheel experiences larger circumference whose radius is larger than the internal wheel thus its speed must be higher than the internal one’s to ensure no wheel slipping occurs. Ackermann geometry is the practical measure to avoid sliding tires. Let’s say we turn right; the electronic differential keeps the left wheel moving at faster rate than the right wheel. Thus, we can express the following parameters as shown in handwritten page image. |
| --- | --- |

## Explanation for chosen default values

|  | We need the values of wheelbase, track width, tire radius, speed of the vehicle COM or angular speed of the system with respect to center of rotation. I had kept my motors on default range of RPM even though they are run on 48V but still as it would be easier to show and simulate, as for a 48V DC motor optimum RPM is nearly 3000 RPM or above, so on sticking with +180 to -180 RPM, we shall assume the values of the unknowns such that our maximum speed for motor be nearly 180 RPM when turning at max steering angle in opposite direction, and near to 0 RPM when turning at max steering angle in the same direction of tire. And here I have set the range of steering angle as -45° to 45°, as a practical range of it is near to 30°. So, in order to get maximum RPM of 180 when at maximum steering angle (45°) of opposite tire and minimum RPM of 0 for same side tire, we shall need to have angular speed of maximum wL = 180.0*2.0*PI/60.0 = 18.85 and minimum wR = 0.0*2.0*PI/60.0 = 0.0, and for making the minimum zero we need to have L – (d/2)*tan(45°) = 0, thus L = d/2. Thus, we now need to find appropriate wheelbase and trackwidth, thus after hit and trial of a range of values, we could approximately say that a system of wheelbase 0.15m, trackwidth 0.30m, tire radius 0.06m and a velocity of 0.56 m/s approximates to nearly 180 RPM in calculation (in simulation it reaches around 178 RPM which is still acceptable). |
| --- | --- |

## Explanation for the Code

|  | I have included math.h library to perform tan function on steering angle. I have used #define to name the pins here also. And stored the wheelbase (L=0.15m), trackwidth (d=0.30m), tire radius (r=0.06m), speed of the system’s COM (v=0.56m/s) as float, and also defined variables steerAngle to store the steering angle, wL and wR to store the angular speed of the left and right tires respectively. In setup, I had defined the pins OUTPUT, INPUT and INPUT_PULLUP using pinMode(). In loop, I run the map() to map the analogRead() value of potentiometer, and convert in between -45 to 45 degrees, and then storing it in radAngle float variable. Then I use if else conditions to verify if it’s a left or right turn, and/or if it’s not turning or not. If it turns right, the steering angle would be negative according to our assumption in starting of the formulas for right turn, thus substituting minus of radAngle in there to get the positive magnitude of the angular speed using the above formulas of left and right tires. If it turns left then we may not need to alter anything in above defined formulas and directly use it to find the angular speed of left and right tire. If it doesn’t turn, so the steering angle’s 0°, thus we may directly substitute it in it and get the equal angular speeds by wL = wR = v/r. Next, we convert the angular speed into RPM by multiplying it with 60.0/2*PI. And then converting the RPMs of each tire to PWM values and using analogWrite(), here we map them from 0-180 to 0-255, thus making a linear scale of PWM and RPM approximately, which is already shown in PS1’s report. Next it is the same if else conditions for checking the direction of motors rotations taken the same way from the first problems solution of DC motor controller, and thus operating the two motors in same manner as we did in the first solution using analogWrite() and digitalWrite(). Note: I have not included the speed controller (potentiometer), current sensor (safety mechanism) as it was done in first question. Adding these elements made the circuit and simulation little complex and thus facing error with reasons of CPU excessive load, thus I decided to exclude the safety mechanism and speed controller over here so to be shown the electronic differential simulation with a higher priority. I hope that doesn’t make anything wrong in the solution. Reference for the simulation is in the same folder as its screen recording also. |
| --- | --- |